# Virtual Network Traffic Analysis Report

Prepared by: Farhan Muntasir
 Tools Used: VirtualBox, Ubuntu VM, Kali Linux VM, Wireshark

## 1. Introduction

The goal of this project was to build a virtual network lab using VirtualBox and analyze traffic between multiple virtual machines. The lab focused on capturing and understanding network protocols including ARP, ICMP, DNS, and HTTP using Wireshark.

Two virtual machines were deployed:

- Ubuntu Desktop → Used for Wireshark packet capture

- Kali Linux → Used to generate traffic (pinging, DNS lookups, browsing)

This lab provides practical exposure to packet analysis, network configuration, and protocol behavior.

## 2. Lab Environment Setup

### 2.1 Virtual Machines

Two VMs were configured in VirtualBox:

| VM | OS | Purpose |
| --- | --- | --- |
| Ubuntu | Ubuntu 22.04 | Monitor and capture packets with Wireshark |
| Kali Linux | Kali Live | Generate network traffic for analysis |

### 2.2 Network Configuration

Both VMs used:

- Host-Only Adapter → For internal communication (Ubuntu ↔ Kali)

- NAT Adapter → To reach the internet (used for DNS + HTTP analysis)

### 2.3 Wireshark Installation

On Ubuntu:

sudo apt install wireshark
sudo usermod -aG wireshark $USER
sudo reboot

Wireshark was used to capture traffic on:

- enp0s3 (Host-Only)

- enp0s8 (NAT)

## 3. Captured Protocols and Analysis

# 3.1 ARP Analysis

Process:

- Start Wireshark on Ubuntu

- Select the Host-Only interface

- Apply filter:

arp

- Ping Kali from Ubuntu or vice versa

What happened:

- ARP packets appeared before ICMP.

- “Who has <IP>? Tell <IP>” messages initiated communication.

- Replies provided MAC addresses.

Conclusion:
 ARP resolves IP → MAC, enabling communication on the local network.

# 3.2 ICMP Analysis (Ping Test)

Process:
 On Kali:

ping 192.168.56.x

In Wireshark:

icmp

Observations:

- ICMP Echo Request packets sent from Kali → Ubuntu

- ICMP Echo Reply packets returned from Ubuntu → Kali

- Low latency and zero packet loss

Conclusion:
 ICMP verified successful connectivity between virtual machines.

# 3.3 DNS Analysis (Domain Name Resolution)

Process:

- Capture DNS on NAT interface (enp0s8)

- Apply filter:

dns

- Run:

nslookup google.com

Observations:

- DNS queries sent from Ubuntu to public DNS servers (8.8.8.8, 75.75.75.75)

- DNS responses returned with resolved IP addresses

- Traffic occurred over UDP port 53

Conclusion:
 DNS successfully translated domain names into IP addresses, demonstrating the name resolution process.

# 3.4 HTTP Traffic Analysis

Process:

- Use NAT interface for HTTP capture

- Apply filter:

http

- Visit a non-HTTPS website such as:

http://testphp.vulnweb.com

Observations:

- HTTP GET requests were captured

- HTTP/1.1 responses returned from the server

- Headers such as Host, User-Agent, Accept were visible

- Traffic was unencrypted because HTTP does not use SSL/TLS

Conclusion:
 HTTP traffic shows complete request and response behavior, illustrating why HTTPS is necessary for security.

# 4. Summary of Findings

| Protocol | What Was Learned |
| --- | --- |
| ARP | Handles IP-to-MAC resolution before communication. |
| ICMP | Confirms host reachability and measures round-trip time. |
| DNS | Converts domain names to IP addresses via UDP port 53. |
| HTTP | Sends and receives unencrypted web requests and responses. |

# 5. Conclusion

The virtual network lab was successfully created using VirtualBox with Ubuntu and Kali Linux. Using Wireshark, we captured and analyzed ARP, ICMP, DNS, and HTTP traffic. This project demonstrates essential networking concepts including host discovery, name resolution, connectivity testing, and application-layer traffic analysis.

The skills gained include:

- Packet capture and filtering

- Understanding real network communication flows

- Using Wireshark for protocol-level analysis

- Configuring VMs for realistic network labs